微小药量燃烧热测定装置及几种典型含能材料燃烧热的测量

郭金坤1，金  波1，楚士晋1，彭汝芳1

（1. 西南科技大学环境友好能源材料国家重点实验室，四川 绵阳 621010）

1. 燃烧热测定

CL-20、RDX、DNTF、FOX-7和NQ的测定方法与HMX一致，其测定结果如表S1~S5所示。

表S1 298.15K时CL-20的燃烧热测定结果

Table S1 Combustion results at T = 298.15 K of CL-20

| m / mg | mcot / mg | QHNO3 / J | Qcor / J | A / mV s | -qEms / J g-1 | -ΔU / kJ mol-1 |
| --- | --- | --- | --- | --- | --- | --- |
| 7.274 | 0.082 | 0.328 | 0.08 | 4038.719 | 8207.7 | 3596.6 |
| 6.564 | 0.080 | 0.370 | 0.07 | 3655.677 | 8195.2 | 3591.1 |
| 6.903 | 0.047 | 0.233 | 0.08 | 3787.363 | 8184.6 | 3586.5 |
| 6.823 | 0.066 | 0.304 | 0.08 | 3769.524 | 8183.8 | 3586.1 |
| 6.748 | 0.057 | 0.263 | 0.07 | 3734.659 | 8224.6 | 3604.0 |
| 6.240 | 0.054 | 0.382 | 0.07 | 3462.082 | 8209.0 | 3597.2 |
|  |  |  |  |  | 8200.8±6.5a | 3593.6±2.8a |
|  |  |  |  |  | 8200.8±13.8b | 3593.6±6.0b |

Note: m, the vacuum mass; mcot, the mass of cotton; QHNO3, the energy correction for the nitric acid formation, Qcor, the corrections to the standard condition (wushborn correction); A is the integral area of the heat flow curve, qEMs is the massic energy of combustion; ΔU, the standard molar energy of combustion. auncertainty represents the standard deviation of the mean; buncertainty represents the overall standard deviation.

表S2 298.15K时RDX的燃烧热测定结果

Table S2 Combustion results at T = 298.15 K of RDX

| m / mg | mcot / mg | QHNO3 / J | Qcor / J | A / mV s | -qEms / J g-1 | -ΔU / kJ mol-1 |
| --- | --- | --- | --- | --- | --- | --- |
| 5.061 | 0.162 | 0.382 | 0.05 | 3373.295 | 9501.4 | 2110.5 |
| 5.970 | 0.155 | 0.334 | 0.06 | 3924.694 | 9505.8 | 2111.4 |
| 5.517 | 0.148 | 0.352 | 0.05 | 3648.454 | 9533.2 | 2117.5 |
| 5.372 | 0.174 | 0.346 | 0.05 | 3588.037 | 9538.0 | 2118.6 |
| 4.412 | 0.153 | 0.274 | 0.04 | 2963.028 | 9524.7 | 2115.6 |
| 5.371 | 0.177 | 0.293 | 0.05 | 3585.933 | 9534.3 | 2117.8 |
|  |  |  |  |  | 9522.9±6.4a | 2115.2±1.4a |
|  |  |  |  |  | 9522.9±15.5b | 2115.2±3.4b |

Note: m, the vacuum mass; mcot, the mass of cotton; QHNO3, the energy correction for the nitric acid formation, Qcor, the corrections to the standard condition (wushborn correction); A is the integral area of the heat flow curve, qEMs is the massic energy of combustion; ΔU, the standard molar energy of combustion. auncertainty represents the standard deviation of the mean; buncertainty represents the overall standard deviation.

表S3 298.15K时DNTF的燃烧热测定结果

Table S3 Combustion results at T = 298.15 K of DNTF

| m / mg | QHNO3 / J | Qcor / J | A / mV s | -qEms / J g-1 | -ΔU / kJ mol-1 |
| --- | --- | --- | --- | --- | --- |
| 13.737 | 0.687 | 0.20 | 8792.951 | 9750.6 | 3043.3 |
| 11.012 | 0.448 | 0.15 | 7039.547 | 9732.7 | 3037.7 |
| 11.958 | 0.573 | 0.17 | 7648.632 | 9736.6 | 3038.9 |
| 11.673 | 0.615 | 0.17 | 7477.15 | 9744.0 | 3041.2 |
| 13.345 | 0.585 | 0.19 | 8515.083 | 9724.1 | 3035.0 |
| 12.161 | 0.549 | 0.17 | 7790.348 | 9755.9 | 3044.9 |
|  |  |  |  | 9740.7±4.8a | 3040.2±1.5a |
|  |  |  |  | 9740.7±15.3b | 3040.2±4.8b |

Note: m, the vacuum mass; mcot, the mass of cotton; QHNO3, the energy correction for the nitric acid formation, Qcor, the corrections to the standard condition (wushborn correction); A is the integral area of the heat flow curve, qEMs is the massic energy of combustion; ΔU, the standard molar energy of combustion. auncertainty represents the standard deviation of the mean; buncertainty represents the overall standard deviation.

表S4 298.15K时FOX-7的燃烧热测定结果

Table S4 Combustion results at T = 298.15 K of FOX-7

| m / mg | QHNO3 / J | Qcor / J | A / mV s | -qEms / J g-1 | -ΔU / kJ mol-1 |
| --- | --- | --- | --- | --- | --- |
| 10.421 | 0.555 | 0.11 | 5597.834 | 8143.4 | 1206.0 |
| 10.485 | 0.501 | 0.11 | 5645.910 | 8169.6 | 1209.9 |
| 10.604 | 0.543 | 0.11 | 5698.255 | 8150.1 | 1207.0 |
| 9.921 | 0.454 | 0.10 | 5351.397 | 8181.7 | 1211.7 |
| 9.953 | 0.561 | 0.10 | 5355.440 | 8150.9 | 1207.1 |
| 9.918 | 0.436 | 0.10 | 5330.419 | 8153.3 | 1207.5 |
|  |  |  |  | 8158.1±5.9a | 1208.2±0.9a |
|  |  |  |  | 8158.1±15.8b | 1208.2±2.3b |

Note: m, the vacuum mass; mcot, the mass of cotton; QHNO3, the energy correction for the nitric acid formation, Qcor, the corrections to the standard condition (wushborn correction); A is the integral area of the heat flow curve, qEMs is the massic energy of combustion; ΔU, the standard molar energy of combustion. auncertainty represents the standard deviation of the mean; buncertainty represents the overall standard deviation.

表S5 298.15K时NQ的燃烧热测定结果

Table S5 Combustion results at T = 298.15 K of NQ

| m / mg | QHNO3 / J | Qcor / J | A / mV s | -qEms / J g-1 | -ΔU / kJ mol-1 |
| --- | --- | --- | --- | --- | --- |
| 10.498 | 0.531 | 0.09 | 5743.64 | 8302.1 | 897.5 |
| 11.447 | 0.675 | 0.10 | 6285.317 | 8330.6 | 900.5 |
| 10.536 | 0.651 | 0.09 | 5782.462 | 8317.7 | 899.1 |
| 12.083 | 0.818 | 0.10 | 6612.097 | 8297.6 | 897.0 |
| 11.957 | 0.567 | 0.10 | 6541.83 | 8315.4 | 898.9 |
| 11.718 | 0.657 | 0.10 | 6410.155 | 8303.9 | 897.7 |
|  |  |  |  | 8311.2±5.0a | 898.4±0.5a |
|  |  |  |  | 8311.2±18.6b | 898.4±2.0b |

Note: m, the vacuum mass; mcot, the mass of cotton; QHNO3, the energy correction for the nitric acid formation, Qcor, the corrections to the standard condition (wushborn correction); A is the integral area of the heat flow curve, qEMs is the massic energy of combustion; ΔU, the standard molar energy of combustion. auncertainty represents the standard deviation of the mean; buncertainty represents the overall standard deviation.

2. 气体产物分析

待测定结束，通过集气袋收集氧弹内气体，并通过气相色谱仪（岛津GC2014）进行分析。以氦气为载气，CO2，O2，N2，N2O，和CO由TCD1检测器进行检测，保留时间（min）: CO2, 11.395; N2O, 12.178; O2, 14.408; N2, 16.398 ; CO, 25.878.

图S1 燃烧后气体产物气相色谱

Fig.S1 Gas chromatography of combustion products